Exp no:8

Determination and plotting of Magnetic Field Intensity, Intrinsic Impedance and Phase Velocity in different medium.

Aim:

To write program for determination and plotting of Magnetic Field Intensity, Intrinsic Impedance and

Phase Velocity in different medium using MATLAB.

Software required:

•MATLAB (R2018 or later)

Test case (i) Determination and plotting of Magnetic Field Intensity due to infinite long conductor at a distance r

Program:

clc;

clear;

I = input('Enter the current (A):');

r = input('Enter the distance (m):');

mu0 = 4*pi*1e-7;

H = I/(2*pi*r);

disp('Magnetic Field Intensity is');

disp(H);

% Plot

x = linspace(0.1,10,100);

H_plot = I./(2*pi*x);

figure;

plot(x,H_plot,'LineWidth',2);

xlabel('Distance (m)');

ylabel('Magnetic Field Intensity (A/m)');

title('Magnetic Field Intensity vs Distance');

Test case (ii) Determination and plotting of Magnetic Flux Density in free space

clc;

I = 5;

mu0 = 4*pi*1e-7;

r = linspace(0.1,10,100);

H = I./(2*pi*r);

B = mu0 * H;

plot(r,B,'LineWidth',2);

xlabel('Distance (m)');

ylabel('Magnetic Flux Density (T)');

title('Magnetic Flux Density vs Distance');

Test case (iii) Determination and plotting of intrinsic impedance for different media

clc;

mu0 = 4*pi*1e-7;

epsilon0 = 8.854e-12;

er = linspace(1,5,100);

ur = linspace(1,2,100);

eta = sqrt((mu0.*ur)./(epsilon0.*er));

plot(er,eta,'LineWidth',2);

xlabel('Relative permittivity');

ylabel('Intrinsic Impedance (Ohms)');

title('Intrinsic Impedance vs Permittivity');

Result :

Thus the program was verified for magnetic field intensity, intrinsic impedance and phase velocity in different medium using MATLAB was successfully.